FTGT - FortiGate

Troubleshoot

! Ping

execute ping

you can use ping-options to choose the source

execute ping-options source x.x.x.x

execute ping-options repeat-count xxx

execute ping x.x.x.x

ICMP protocol 1

Entering the correct vdom/global config

# config global

# sudo root execute ping 10.71.224.44

# config vdom

(vdom) # edit root

(root) # execute ping 10.71.224.44

! ARP

get system arp

get system arp | grep 10.100.230.1

get system arp | grep ec:e5

get system arp | grep wan/mac

! DNS

execute nslookup (is not always working, check)

diagnose firewall fqdn list

Routing

! Routing table - RIB

dia

get router info routing-table detail

show system route

! Routes configured.

Only active routes show the > symbol, which mean they are selected and active routes. The routing table database contains all active, standby, and inactive routes on FortiGate.

get router info routing-table database

! Routing table, only active routes

get router info routing-table all

! To change ECMP load balance method, by default is source IP address

! To archive ECMP with static routes, the distance and priority values must be the same for all routes.

config system settings

set v4-ecmp-mode source-dest-ip-based

end

Verify using:

HQ-NGFW-1 # get router info routing-table all

HQ-NGFW-1 # diagnose sniff packet any 'tcp[tcpflags] & tcp-syn !=0 and tcp[tcpflags] & tcp-ack == 0 and port 80' 4

The 'tcp[tcpflags] & tcp-syn !=0 and tcp[tcpflags] & tcp-ack == 0' filter matches packets with the SYN flag on, so the output will show only all SYN packets for port 80 (HTTP).

! Policy routes.

diagnose firewall proute list

# config system route

(route)# edit 0

(0)# set dst 0.0.0.0/0

(0)# set gateway 10.252.143.17

(0)# set device port1

(0)# end

show system route

# get router info routing-table all

# config system interface

# set ip ______ ________

# config router static

# show

# edit __

# show system interface | grep -f 10.112.254.19

Packet Capture (Sniffer)

# diagnose sniffer packet <interface_name> <‘filter’> <verbose> <count> <tsformat>

# diagnose sniffer packet any none 4 10

<verbose>

1 - print header of packets

2 - print header and data from IP of packets

3 - print header and data from Ethernet of packets

4 - print header of packets with interface name

<count>

The number of packets the sniffer reads before stopping. If you don't put a number here, the sniffer will run until you stop it with < CTRL+C >.

# diagnose sniffer packet port1 none 1 3

FGT# diagnose sniffer packet any "host <PC1> or host <PC2>" 4

# diagnose sniffer packet any "host 10.35.50.155 and host 10.100.21.239" 4

FGT# diagnose sniffer packet any "(host <PC1> or host <PC2>) and icmp" 4

FGT# diagnose sniffer packet any "host <PC1> or host <PC2> or arp" 4

# diagnose sniffer packet VENDOR_DMZ 'host 255.255.255.255 and udp and port 67 or port 68' 6 100

diagnose sniffer packet any 'host 174.211.160.125 and host 10.154.0.30' 4

diagnose sniffer packet any 'host 73.139.19.48 and host 10.154.0.30' 4

diagnose sniffer packet any 'host 10.154.0.30' 4

Add 100 al final para que la captura pare en 100 lineas

# diagnose sniffer packet any 'port 80' 4 10

# diagnose sniffer packet any 'portrange 80-443' 4 10

# diagnose sniffer packet any 'host x.x.x.x' 4 10

# diagnose sniffer packet any 'src x.x.x.x' 4 10

# diagnose sniffer packet any 'dst x.x.x.x' 4 10

# diagnose sniffer packet any 'arp' 4 10

# diagnose sniffer packet any 'icmp' 4 10

# diagnose sniffer packet any 'tcp' 4 10

# diagnose sniffer packet any 'less 64' 4 10 ! Packet size

# diagnose sniffer packet any 'src x.x.x.x and dst port 80' 4 10 ! Packet size

set advanced-filter "net 172.16.200.0/24 and port 443 and port 49257"

Debug Flow

diagnose debug flow trace stop

diagnose debug enable

diagnose debug flow filter addr 10.154.0.30

diagnose debug flow show function-name enable

diagnose debug flow trace start 10

Diagnose Session

diagnose sys session filter src 10.154.0.30

diagnose sys session filter proto 1

diagnose sys session filter

diagnose sys session list

diagnose sys session stat

diagnose sys session clear

Configure the FortiGate to send TCP RST packet on session timeout

https://kb.fortinet.com/kb/documentLink.do?externalID=FD35049

#config firewall policy

# edit <ID>

#  set timeout-send-rst enable

Technical Tip: Session timeout settings

https://kb.fortinet.com/kb/documentLink.do?externalID=FD40914#:~:text=It%20is%203%2C600%20seconds%20by,is%20300%20seconds%20by%20default.

Technical Note: Change session ttl on firewall policy

https://kb.fortinet.com/kb/documentLink.do?externalID=FD35176

# config firewall policy

# edit [rule number]

# set session-ttl [seconds]

# end

dia sys session list

Restarting Logging

#fnsysctl killall miglogd

If you are looking to troubleshoot the logging issue, you can also dig into the miglogd debug itself:

#dia de app miglogd -1

#dia de en

Show running-config & grep & scp

# show full-configuration

# show full-configuration | grep -f VLAN_112

! To check where the users are connected:

! SSH to the fortigate:

diag debug auth fsso list | grep BACKUP57

! On windows to check to wat DC are you loging:

echo %logonserver%

get sys status

Diag debug flow filteraddress

FW01 # dia hardware deviceinfo nic port2

FW01 # diagnose netlink interface list port1

FW01 # config vdom

FW01 (vdom) # edit root

FW01 (root) # get router info routing-table all

To check NAT exhaust, check counter clash=

HOM6-C-0102-FW01 $ diagnose sys session stat

! to view the port3 interface configuration

# show system interface port3

! The show full-configuration command displays all the configuration settings for the interface.

# show full-configuration system interface port3

?????

execute central-mgmt register-device FMG-VM0A17005520 pwd123

FAZ - FortiAnalyzer

! To rebuild DataBase

execute sql-local rebuild-db

! To schedule start-time

https://community.fortinet.com/t5/FortiAnalyzer/Technical-Note-FortiAnalyzer-SQL-database-rebuild-start-time/ta-p/192363

! To get the TAC support file

execute tac report

Bandwidth Report

FortiAnalizer (Ship) > FortiView > Traffic

FortiGuard DNS

208.91.112.53

208.91.112.52