# Guest Portal SSID Configuration: Aruba WLC and Cisco ISE

Step-by-step technical documentation

# 1. Document Control

| Document Title | Guest Portal SSID Configuration: Aruba WLC and Cisco ISE |
| --- | --- |
| Prepared By | [Alejandro Power / Shipboard Network Operations] |
| Version | [1.1] |
| Date | [6/22/2026] |
| Environment | [GSC] |
| Change / Request Reference | [CHG0104481] |

# 2. Purpose

This document provides a step-by-step template for configuring a guest portal SSID using Aruba Wireless LAN Controller and Cisco ISE. The intent is to document the configuration workflow, capture screenshots for each step, and provide a repeatable reference for deployment, validation, and troubleshooting.

# 3. Scope

- Configure Cisco ISE guest portal components.

- Configure Aruba WLC RADIUS integration and captive portal redirection.

- Create or update the guest SSID profile.

- Create firewall rules, interfaces and dns services in the Fortigate

- Validate guest onboarding, portal redirection, authentication, authorization, and internet access.

- Document screenshots for each configuration step.

# 4. Prerequisites

- Administrative access to Cisco ISE.

- Administrative access to Aruba WLC or Mobility Conductor.

- Guest VLAN created and routable as required.

- DHCP scope configured for the guest VLAN.

- DNS reachability available for guest clients.

- Firewall rules configured for required guest access, Cisco ISE portal access, RADIUS, and CoA.

- Valid certificate installed for the Cisco ISE guest portal to avoid browser trust issues.

- Aruba WLC running a supported ArubaOS version for the required captive portal and RADIUS attributes.

# 5. Network and Configuration Details

| Item | Value | Notes |
| --- | --- | --- |
| SSID Name | [NCL_GUEST] | [Guest-WiFi] |
| Guest VLAN | [66] | [10.142.66.1/23] |
| Aruba WLC IP Address | [10.142.66.2] | [10.142.0.250] |
| Cisco ISE PSN | [10.142.0.110-10.142.0.109] | [Portal and RADIUS node details] |
| RADIUS Shared Secret | [Standard password] | Fleet standard configuration |
| RADIUS Auth Port | 1812 | N/A |
| RADIUS Accounting Port | 1813 | N/A |
| CoA Port | 3799 | [Confirm allowed between ISE and Aruba WLC] |
| Guest Portal FQDN | [gscguest.nclmiami.com] | [Certificate should match portal FQDN] |

# 6. Cisco ISE Configuration

## 6.1 Add or Verify Aruba WLC as a Network Device

- Navigate to Administration > Network Resources > Network Devices.

- Select Add or open the existing Aruba WLC device entry.

- Enter the Aruba WLC name, IP address, device type, location, and RADIUS shared secret.

- Enable RADIUS authentication settings.

- Assign the Aruba network device profile if required for Aruba-specific attributes and CoA behavior. SUPER IMPORTANT WITHOUT THIS CONFIG THE USERS WONT CHANGE THE ROLE AFTER AUTHENTICATION.

- Save the configuration.

## 6.2 Configure Guest Portal

- Navigate to Work Centers > Guest Access > Portals & Components > Guest Portals.

- Create or select the required guest portal type, such as Hotspot, Self-Registered Guest, or Sponsored Guest.

- Configure the portal name, HTTPS port, allowed interfaces, certificate, login behavior, AUP settings, and guest user flow.

- Confirm the portal FQDN and certificate are valid and trusted by guest endpoints.

- Save the portal configuration.

## 6.3 Configure Authorization Profiles

- Navigate to Policy > Policy Elements > Results > Authorization > Authorization Profiles.

- Create a pre-authentication authorization profile for guest redirection.

- Configure the profile to redirect unknown or unauthenticated guest clients to the Cisco ISE guest portal.

- Configure the redirect ACL or Aruba role name expected by the Aruba WLC.

- Create a post-authentication authorization profile for successfully authenticated guest users.

- Assign the final guest VLAN, Aruba user role, or access result as required by the deployment design.

- Save the authorization profiles.

## 6.4 Configure Policy Set

- Navigate to Policy > Policy Sets.

- Create or update the policy set matching Aruba wireless guest traffic.

- Define the conditions using network device, device type, SSID, NAS-ID, or other available attributes.

- Configure authentication policy for MAB or guest flow as required.

- Configure authorization rules for unknown guest endpoints, guest portal redirection, and completed guest access.

- Save and validate the policy set order.

# 7. Aruba WLC Configuration

## 7.1 Create or Update Guest SSID

- Navigate to the WLAN or SSID configuration section.

- Create a new WLAN or edit the existing guest SSID.

- Set the SSID name and primary usage as Guest.

- Select the AP group or deployment scope where the SSID should be broadcast.

- Assign the guest VLAN or VLAN pool.

- Configure the forwarding mode based on the deployment design.

- Enable captive portal or internal captive portal as required so Aruba can perform client redirection.

- Assign the Cisco ISE RADIUS server group.

- Assign the pre-authentication role and final guest role as required.

- Save, deploy, and commit pending changes.

- Navigate to Configuration > Authentication > Auth Servers.  Click the + button under All Servers.  Fill in name, select type as RADIUS, and fill in the IP address/hostname of the ISE PSN.  Click Submit.  Repeat for each of the ISE PSNs.

- Select the newly created RADIUS Server definition. Enter the Shared Key and click Submit.  Repeat for each of the ISE PSN RADIUS Server definitions.

- Navigate to Configuration > Authentication > AAA Profiles.  Select the AAA profile for the newly created WLAN, [SSID]_aaa_prof.  Enable RADIUS Interim Accounting and DHCP Enforce. Click Submit.

- Select RADIUS Accounting Server Group. Change Server Group to the ISE Server Group created previously.  Click Submit.

- Select MAC Authentication. Change MAC Authentication Profile to Default. Click Submit.

- Select RFC 3576 Server.  Click the + button and select the ISE PSN from the drop down.  Click Submit.  Repeat for each of the ISE PSN RFC 3576 server definitions.

- Click Pending Changes in the top right and click Deploy changes to deploy the configuration to the Mobility Controller.

### Role & Policy Configuration

- Navigate to Configuration > Roles & Policies > Policies and click the + button.

- Select the newly created policy and click the + button.  Select Access Control and click OK. Create a new forwarding rule allowing captive portal traffic to the ISE PSNs.  Click Submit.

- Select the newly created Role from the list.  Click Show Advanced View.

- Click the + button within Policies. Select Add an existing policy. Select type Session and select the policy created in the previous step.  Click Submit.

- Select the Captive Portal tab. And make sure configuration is in place.

- Click Submit.

- Click Pending Changes in the top right and click Deploy changes to deploy the configuration to the Mobility Controller.

Note: L3 configuration is needed for the guest VLAN in order to work with the guest portal redirection :

# Firewall Configuration

- Configure interface guest VLAN with the firewall as primary dns server.

- Navegate to interfaces > DNS Databases

- Create new DNS Database with the A records for guest portal FQDN, sponsor FQDN so the clients can resolve it to the ISE server IP address.

- Create the necessary firewall rules for guest traffic to internal network and internet access.

# 8. Validation Procedure

- Connect a test client to the guest SSID.

- Confirm the client receives an IP address from the guest VLAN DHCP scope.

- Open a browser and confirm redirection to the Cisco ISE guest portal.

- Complete the guest portal login, registration, or AUP acceptance flow.

- Confirm Cisco ISE sends the expected authorization result and CoA.

- Confirm the client is moved to the final guest role or VLAN.

- Validate DNS resolution and internet access.

- Verify Cisco ISE RADIUS Live Logs show successful authentication and authorization.

- Verify Aruba WLC client details show the expected role, VLAN, IP address, and session state.

# 10. Troubleshooting Checklist

| Issue | Validation | Action |
| --- | --- | --- |
| No IP address | Check VLAN, DHCP scope, helper address, and client VLAN assignment. | Correct VLAN or DHCP configuration. |
| No portal redirect | Check pre-auth role, redirect profile, DNS, portal FQDN, and allowed ACLs. | Allow DNS, DHCP, and ISE portal access; verify redirect authorization profile. |
| ISE authentication failure | Review Cisco ISE RADIUS Live Logs. | Confirm network device, shared secret, policy set, and endpoint matching conditions. |
| CoA failure | Check CoA port reachability and Aruba device profile settings. | Allow UDP 3799 and confirm CoA support. |
| Certificate warning | Check portal certificate CN/SAN and trust chain. | Install or bind a trusted certificate for the guest portal FQDN. |
| Guest has no internet after login | Check final role, VLAN, firewall policy, NAT, and DNS. | Correct post-auth role and upstream firewall/NAT policy. |

# 12. Implementation Notes

- [GUEST traffic is configured on SD-WAN as local breakout.]

- [Certificate needs to be renewed every 6 months for now]

- [Once users are moved to the new SSID, old SSID with mac authentication will be disabled]