Riverbed 1st Time Setup Tasks

========================

# Configure IP Primary interface (PRI)

The Primary interface (PRI) this is the management interface for Riverbed Steehead, this task is executed in the CLI; and you must configure PRI IP, Mask, and Gateway. The rest of the configuration can be executed by GUI.

There are two ways to configure it, following the wizard for initial configuration or adding the commands directly to the running configuration.

following the wizard for initial configuration

Power up the SteelHead and connect a serial cable from your laptop to the console port of the appliance.  Follow the steps below in the Config wizard (the following is an example), important to answer NO in Step 2:

Riverbed SteelHead

amnesiac login: admin

Password: password

Last login: Tue Dec 13 21:37:26 on ttyS0

Riverbed SteelHead configuration wizard.

Do you want to auto-configure using a CMC? no

Do you want to use the wizard for initial configuration? yes

Step 1: Hostname? [amnesiac] JOYRVBD01

Step 2: Use DHCP on primary interface? [no]

Step 3: Admin password? Bug…

Step 4: SMTP server? []

Step 5: Notification email address?

Step 6: Set the primary interface speed? [auto]

Step 7: Set the primary interface duplex? [auto]

Step 8: Would you like to activate the in-path configuration? [no]

Step 9: Would you like to activate the out-of-path configuration? [no]

You have entered the following information:

1. Hostname: JOYRVBD01

2. Use DHCP on primary interface: yes

3. Admin password: …

4. SMTP server:

5. Notification email address:

6. Set the primary interface speed: auto

7. Set the primary interface duplex: auto

8. Would you like to activate the in-path configuration: no

9. Would you like to activate the out-of-path configuration: no

To change an answer, enter the step number to return to.

Otherwise hit <enter> to save changes and exit.

Choice: <enter>

Adding the commands directly to the running configuration

no interface primary dhcp

interface primary ip address pri.ip.address /mask

ip default-gateway gateway

# Host Setting

In this step you will configure Hostname, DNS, DNS Domain List. This step is important for the steelhead to have DNS to resolve for the Riverbed Licensing Portal. You must consult server team ship/shoreside to confirm this information if needed.

Open the SteelHead GUI: https://pri.ip.address

Sign in to the GUI as admin

Networking > Host Setting

Hostname:

Primary DNS Server:

Secondary DNS Server:

Tertiary DNS Server:

DNS Domain List:

The following is the DNS Settings for nclmiami domain:

DNS Servers: 167.118.162.155, 167.118.162.156,

10.100.20.90, 10.100.20.91, 10.100.20.92

Domain List: nclmiami.ncl.com, ncl.com

Test DNS at the end of the page using nclmiami.ncl.com or the ship domain.

Save Configuration

# ADMINISTRATION

## Licensing

ADMINISTRATION > Maintenance > Licenses

Before performing any other task, is mandatory to license the device.

First you need to setup PRI (Base Interface) IP and DNS to automatically license the device. If the PRI address is allowed to go to internet the steelhead will update licenses automatically Fetch Update Now bottom.

If automatically licensing doesn’t work; you must follow the guide to manually licensing the device.

## Upgrade

ADMINISTRATION > MAINTENANCE > Software Upgrade

There is a strict order to follow when upgrading the software, depending on the current version on the appliance.  You must look up the sequence for the destination RiOS version and the version intallaed, Riverbed support site.  You must follow the correct sequence.

Some RiOS software are in \\netsrv1\wwwroot\RIVERBED folder.

Versions:

The following are the standard that we want the get; not all the devices support these versions as they are End of Software (EoS), for those ones upgrade to the latest supported.

Shipside: RiOS 9.7.1 ----> 9.9.1

Shoreside: RiOS 9.8.1 ----> 9.9.1

## Date/Time

ADMINISTRATION > SYSTEM SETTING > Date/Time

Time Zone: US/Eastern

Use NTP Time Synchronization

When you change the Time Zone, you will be asked to re-Sign in.

NTP Servers

- Disable all default NTP servers, do not delete it.

- Datacenters 10.100.0.12, 10.102.0.12

- Ships: ntp.shipname.ncl.com

## SNMP – Basic

ADMINISTRATION > SYSTEM SETTING > SNMP Basic

Enable SNMP Traps

System Contact: Global Network Engineering

System Location: Site/Ship Name to match the Solarwinds Sites

Read-only Community string: nclpub

## EMAIL

ADMINISTRATION > SYSTEM SETTING > Email

SMTP Server: forwarder.shipname.domain.com

Report Events: ShipboardNetworkEngineering@ncl.com, global-network-engineering-ops@ncl.com

Report Failures: ShipboardNetworkEngineering@ncl.com, global-network-engineering-ops@ncl.com

Override Default Sender’s Address: global-network-engineering-ops@ncl.com

Report Failures to Technical Support

Create ticket to server team asking to use the forwarder.

global-network-engineering-ops@ncl.com,ShipboardNetworkEngineering@ncl.com

167.118.92.200 -- > forwarder.nclmiami.ncl.com

## LOGGING

ADMINISTRATION > SYSTEM SETTING > Logging

Add Log Servers:

167.118.105.66 (only for NCL.COM Sites)

167.118.105.67 (only for NCL.COM Ships)

167.118.105.65 (only for PCH Ships)

Severity: Warning

## Web Settings

ADMINISTRATION > SECURITY > Web Setting

Default Web Login:

Web inactivity timeout: 30

Check: Allow Session Timeouts when viewing Auto-Refreshing Pages

Generate a new Self-Signed Certificate

Organization Name: Norwegian Cruise Line

Organization Unit Name: Global Network Engineering

Locality: Miami

State: FL

Country: US

Email Address: global-network-engineering-ops@ncl.com

Validity Period: 3650

## Security

(No implemented)

ADMINISTRATION > SECURITY > TACACS+

General Settings:

Select:	TACACS+; Local

Uncheck: For RADIUS/TACAS+, fallback only when servers are unavailable

- Configure “NETWORKING”

Base Interface

This is the management IP address

Configure Primary interface as needed.

In-Path Interface

Configure In-Path interface as needed.

Flow Statistics (NetFlow) _ This is the default

Check Enable Application Visibility

Check WAN Throughput Statistics

Check Enable Top Talkers – 24h

- Configure “OPTIMIZATION”

NETWORK SERVICES

General Service Settings

Enable In-Path Support (requires an in-path IP first)

Peering Rules

Check:	Enable Enhanced Auto-Discovery

Transport Settings

TCP Optimization

Ships: Bandwidth Estimation

Buffer Settings

Ships: 1000000 LAN / 3000000 WAN

Default: CX755L

Failover Setting

In case of two steelhead in the ship we can enable Failover Support; this will sync data storage and connections between both devices. In case of a traffic failover the new active steelhead will have all the cache. One of the devices is the Master and other the Backup.

OPTIMIZATION > NETWORK SERVICES > General Service Setting

Failover Setting

Enable Failover Support

Current Appliance is:	Master/Backup

IP Address: 			peer in-path interface

SSL

Certificate Authorities

Remove all pre-installed CA’s this will avoid future alarms due to expiring certificates.

Add “Entrust CA” certificate from \\wwwroot\Riverbed\Certificates

SSL Main Settings

Enable SSL Optimization

Add NCL.com and RSSC.com root certs

SECURE PEERING

Generate a new Self-Signed Certificate(replace)

Norwegian Cruise Line

Global Network Engineering

Miami

Florida

global-network-engineering-ops@ncl.com

PROTOCOLS

SMB2/3

Enable SMB2/SMB3 optimizations

MAPI

Enable encrypted optimizations (NTLM Transparent Mode)

HTTP

Enable:

Stream Splitting

SharePoint: FPSE, WebDAV

## Optimization Rules

FortiAnalyzer

This is an encrypted traffic that riverbed is unable to optimize, create a passthrough.

- Create Host Label FortiAnalyzer 10.100.21.115/32

- Create passthrough rule to destination 10.100.21.115/32 port: 3000 Description: Shoreside FortiAnalyzer

! Depending of the version, you can setup the rule using destination the Host Label.

AD Replication

Solution Number: S20240

Active Directory replication traffic between Windows Domain Controllers is both compressed and encrypted. There is no way for the SteelHead to decrypt this traffic.

- Create a Port Label: AD-RPC 135, 49152-65535

Host Label for the Domain Controllers

Ship domain controller NCL-shipname-ADs

seaweb

167.118.162.125/32

167.118.162.178/32

167.118.191.4/32

SSL seaweb.ncl.com

!To review:on Jade review PassThrough_Ports label.

Host Label without_steelhead

To avoid optimization to destination without steelhead.

To Site No Riverbed

- Additional Tasks

Configure for TACACS+

Once the system goes into production, the Self-Signed Peer Gray-List must be reviewed to move peers to the white-list.

For all peers in the Gray-List, under Actions, select Trust.

Configuring QOS

- Create Network Applications

  - Goto Networking, Applications

  - Create all of the following applications:

APP-CITRIX-FARM-MIA-T2

10.100.48.0/23

Custom Application

Other

Medium Critically

APP-CITRIX-FARM-SMY-T1

10.106.48.0/23

Other

Medium Critically

APP-CITRIX-FARM-SMY-T2

10.102.48.0/23

Other

Medium Critically

APP-ELAVON

198.203.192.219 fusebox.elavon.net

198.203.192.219/32

Web Services

Highest Critically

APP-EVERI-ATM

173.226.121.5 BARRACUDA VPN HUB

Other

Highest Critically

APP-EVERI-CASHCLUB

173.226.121.18 xview.gcacashclub.com

173.226.121.18/32

Web Services

Highest Critically

APP-NOVATE

162.13.108.124 fexcogateway.fexcodccapps.com

162.13.108.124/32

Web Services

Highest Critically

ICMP

Transport Layer Protocol: ICMP

Network Monitoring

Medium Critically

SHIP-NET-MGMT

10.X.0.0/24

Networking

Medium Critically

seaweb

seaweb.ncl.com

seaweb.ncl.com/*

Web Services

Highest Critically

fexcogateway

fexcogateway.fexcodccapps.com

162.13.109.113

162.13.108.124

Web Services

Highest Critically

  - IP Addresses for all APPS should be in the destination field, except for SHIP-NET-MGMT, where the IP address should be in the SOURCE field.

- Create QOS Profile “Marking”

  - Goto Networking, Quality of Service

  - Under QOS Profiles, Add a QoS Profile named “Marking”

  - Add all of the following Applications to the profile

- Go to Networking, Sites & Networks

- Add a network

Name: VSAT

- Edit site Default Site

  - Rename to SHORESIDE”

  - Select Outbound QoS Profile: “Marking”

- Edit site “Local Site”

  - Add only the ship subnets, like 10.X.0.0/16

  - Some ships can have others old subnets.

- Goto Networking, Quality of Service

  - Enable Outbound QoS Marking

  - Click “Save”

- Test by pinging a 10.x.0.x host in the ship and ensure replies come back tagged as AF21

In case two riverbed in Ship

Add to Riverbed Asset

QoS

Solarwind

Block HTTP mgmt only HTTPS

DNS (to document)

10.X.50.24

10.X.50.26

Upgrade the Software to RiOS 9.7.1 (if supported)

NTP

SMTP relay (Email)

SMB signed

Domain join

Add steelhead to the CMC

SSL/TLS1.2

Citrix Optimization

Independent Computing Architecture (ICA)

Citrix ICA Citrix ICA ports 1494 and 2598